50  Kings Park Road, West Perth
WESTERN AUSTRALIA, 6005

FORRESTANIA

EXPLORATION ENVIRONMENTAL
MANAGEMENT PLAN

(Western Australia)
Rev-1  June 2026

TABLE OF CONTENTS

1.0	Introduction	4

1.1	Purpose	4

1.2	Location and Access	4

1.3	History	5

1.4	LEGAL OBLIGATIONS	5

1.5	DEFINITIONS	6

2.0	Exploration Activities / Methodology	8

2.1	Geochemical soil sampling	8

2.2	Geological mapping and rock chipping	9

2.3	Ground gravity geophysical surveying	9

2.4	Down Hole Electromagnetic (EM) Surveys	10

2.5	Drilling	11

2.6	Clearing and disturbance	12

2.7	Camping	12

2.8	Hydrocarbons and other chemicals	13

3.0	ENVIRONMENTAL RISK ASSESSMENT	14

4.0	ENVIRONMENTAL RISK MANAGEMENT	20

4.1	Access and safety	20

4.2	Weed and dieback management	21

4.3	General waste management	22

4.4	Spill management	22

4.5	Flora and fauna management	23

4.6	Water management	26

4.7	Fire management	26

4.8	Dust, light and noise management	26

4.9	Heritage and culture management	27

4.10	Induction and training	27

4.11	Incident reporting	28

5.0	ROLES AND RESPONSIBILITIES	29

6.0	REPORTING AND COMMUNICATION	30

6.1	Internal	30

6.2	External	30

7.0	REFERENCES	32

8.0	Revision Tracking	34

List of Tables

Table 1.  Potential Hazard Pathway from exploration activities	15

Table 2.  Relative risk of exploration activities (DBCA 2011)	16

Table 3.  Management requirements as determined by risk ranking.	17

Table 4.  Inherent and residual risk of proposed exploration phases.	18

Table 5.  Summary of risk management sections.	20

Table 6.  External reporting requirements.	31

List of Figures

Figure 1.  FGP Project Area	5

Figure 2.  Soil sampling techniques	9

Figure 4.  Representative photos of how EM surveying is conducted.	10

Figure 5.  A Diamond Drill Rig set up at an Exploration site.	11

Figure 6.  Access track clearance in Medallion Exploration	12

Introduction

Purpose

This document presents an Exploration Environmental Management Plan (EEMP) for exploration activities managed by Medallion Metals (Medallion) or other parties on Medallion owned tenements at the Forrestania Gold Project (FGP), Western Australia.

The EEMP briefly describes the potential environmental risks associated with exploration and details the management programs proposed by Medallion to mitigate potential impacts on the local environment arising from disturbance and other activities associated with the proposed exploration programs.

The EEMP fulfils two key purposes:

It outlines the scope of proposed exploration activities, environmental risks and impacts, and the management measures in order to facilitate statutory approval for those activities;

It communicates the relevant environmental information and management measures applicable to exploration activities in Western Australia to Medallion employees, contractors, regulators and other stakeholders.

The implementation of this EEMP will ensure that all exploration activities are:

carried out in an environmentally responsible manner such that any impacts from the activities are avoided, or if unavoidable, are minimised; and

fulfil the obligations under existing tenement conditions and other legislation.

This EEMP is to be used to support any Programme of Work (PoW) applications and native vegetation clearing permit (NVCP) applications submitted to the Department of Mines, Petroleum and Exploration (DMPE).

Location and Access

The EEMP is valid for all exploration activities within the FGP. Figure 1 shows Medallion Forrestania tenure as of June 2026. Access to tenement will utilise established roads and tracks (Figure 1). Where access to new work areas is required, new tracks may need to be created.

Figure 1.  FGP Project Area

History

The Forrestania Gold Project (FGP) encompasses a historically productive tenure that has seen a succession of major and mid-tier mining groups advance its resource potential since the 1990’s.

Following historical operations by Viceroy Resource Corporation around 2000, the project was operated by Sons of Gwalia from 2002 to 2004, before reverting back to LionOre ownership between 2005 and 2006. In late 2006, Kagara Nickel acquired the asset, establishing a split-commodity framework where St Barbara held the gold rights and Western Areas NL retained the base metal rights.

Western Areas subsequently consolidated full ownership of the tenure in 2013, which later passed to IGO Limited following its corporate acquisition of Western Areas.

In February 2026 Medallion Metals Limited acquired 100% of the FGP tenements from IGO, with IGO retaining the nickel and lithium rights.

Medallion Metals are now the FGP Project operators , currently working towards recommencement of the Cosmic Boy Processing facility to treat Gold-Copper ore from the Ravensthorpe Project, whilst also undertaking regional and advanced stage gold exploration and resource development at FGP.

LEGAL OBLIGATIONS

Medallion will comply with the relevant provisions in applicable Acts and their Regulations during exploration activities which include but may not be restricted to the following:

Aboriginal Heritage Act 1972

Conservation and Land Management Act 1984

Contaminated Sites Act 2003

Environmental Protection Act 1986

Environmental Protection Regulations 1987

Environmental Protection (Noise) Regulations 1997

Environmental Protection (Clearing of Native Vegetation) Regulations 2004

Environmental Protection (Rural Landfill) Regulations 2002

Environmental Protection (Unauthorised Discharges) Regulations 2004

Environment Protection and Biodiversity Conservation Act 1999 (Commonwealth)

Dangerous Goods Safety Act 2004

Public Health Act 2016 and, where still applicable during staged implementation, the Health (Miscellaneous Provisions) Act 1911

Mining Act 1978 and Regulations 1981

Work Health and Safety Act 2020 and applicable regulations

Native Title Act 1993 (Commonwealth)

Rights in Water and Irrigation Act 1914

Work Health and Safety Act 2020 and applicable regulations

Biodiversity Conservation Act 2016 and Biodiversity Conservation Regulations 2018

DEFINITIONS

AC – Aircore drilling

DD – Diamond drilling

DBCA – Department of Biodiversity, Conservation and Attractions

DWER – Department of Water and Environmental Regulation

DMPE – Department of Mines, Petroleum and Exploration

Threatened species –  Listed by order of the Minister as Threatened in the category of critically endangered, endangered or vulnerable under section 19(1), or is a rediscovered species to be regarded as threatened species under section 26(2) of the Biodiversity Conservation Act 2016 (BC Act).

EEMP – Exploration Environmental Management Plan

ESA – Environmentally Sensitive Area, as defined by Regulation 6 of the Environmental Protection (Clearing of Native Vegetation) Regulations 2004.

FGP – Forrestania Gold Project

FNO – Forrestania Nickel Operations

JV – Joint venture between Medallion and other parties.

PoW – Programme of Work, an approval granted under the Mining Act 1978 for mineral exploration activities to be undertaken in an approved area.

Priority species – Priority is not a listing category under the BC Act. The Priority Flora and Fauna lists are maintained by the department and are published on the department’s website  (Priority 1 to Priority 4). These taxa are typically considered to require further survey or monitoring as follows:

Priority 1: Poorly-known species - known from few locations, none on conservation lands).

Priority 2: Poorly-known species - known from few locations, some on conservation lands

Priority 3: Poorly-known species - known from several locations

Priority 4: Rare, Near Threatened and other species in need of monitoring RC – Reverse circulation drilling

NVCP – Native Vegetation Clearing Permit.

Medallion – Medallion Metals

Exploration Activities / Methodology

This section describes the main characteristics of the proposed exploration works, focusing on key operational details. Details on how the exploration works will be managed, particularly how environmental impacts are to be avoided or minimised, are provided in Section 4.0.

Details of the proposed exploration activities are preliminary and may change. Final exploration works for each stage of the program involving ground clearing will be detailed in the relevant PoW submission to DMPE.

Geochemical soil sampling

Geochemical soil sampling involves collecting samples of soil from the surface of the Earth and analysing them to determine which chemical elements are present. Geochemical soil sampling is a low impact sampling technique that has the potential to ‘see’ through overlying soil to the rocks below and help target areas of interest and / or prospectivity.

Soil samples are generally collected on a gridded pattern, with closer spaced sample sites along more widely spaced sample lines. A fine fraction (<3mm) soil sample or a coarse fraction (3-15mm) lag sample may be collected depending on soil type and characteristics. Soil samples are collected from near surface i.e., generally <50cm, while lag samples are collected at surface (Figure 2).

The equipment required to collect and prepare geochemical soil samples in the field includes:

GPS to record location of sample site.

Shovel, pick (shallow) or handheld spiral auger (deeper) to collect fine fraction soil sample.

Dustpan and brush or plunger magnets to collect coarse fraction lag sample.

Metal sample scoop to remove soil from excavated sample site.

Sieve and receiving pan to sieve and collect sample.

Chemical-free paper or calico sample bag to contain and store sample.

Large heavy duty plastic bags to pack paper/calico bags into for transport.

The process of collecting and preparing geochemical soil samples is relatively quick (minutes) and has little to no impact, with a small disturbance at the site where the sample is obtained from. The sample location will be selected to ensure no disturbance to vegetation (i.e., taken from an area without vegetation, as close as possible to the sampling point) or potential fauna habitat (e.g., logs or large rocks will not be moved).  At each sampling location, a maximum of 3kg of material will be collected, with sample weights typically 1.5kg. Samples are contained in a paper or calico bag and placed in a heavy duty plastic bag for transport. Any sample holes will then be backfilled and mounded over immediately after the sample is collected.

Once collected and prepared, geochemical soil samples are sent to a laboratory for analysis. Depending on the assay results, small areas of interest may be revisited to collect duplicate samples.

Figure 2.  Soil sampling techniques

Geological mapping and rock chipping

The proposed mapping and rock chip sampling involves sampling of rock chips using a geological hammer on an ad hoc basis where rock outcrops are encountered during mapping traverses.

Typically, 1 kg, but up to 3 kg, of rock will be collected per sample. Samples are collected using a geological hammer to chip the edges of large outcrop exposures. As the location of outcrops is unknown until the mapping exercise is carried out, the quantity of rock chips to be collected is also unknown; however, it is expected to be minimal. Collection will be undertaken to minimise disturbance to potential fauna habitat and flora (for example, by chipping rocks rather than moving or removing large rocks).

Ground gravity geophysical surveying

The gravity method is a passive scientific technique that monitors natural variations in the Earth’s gravity field at different locations. These gravity variations represent changes in the density of rocks below the surface.

Gravity survey equipment typically consists of a gravity meter, a GPS base station and roving GPS unit. At each observation site the gravity meter is lifted out of its box or cradle, placed on the ground or a baseplate, and the meter is levelled and read (optically or digitally). The entire process typically takes only 5-10 minutes per sampling point.

Approximately 1000 ground gravity stations on a 200-400m grid are proposed to be collected within the SWT project area on DBCA lands.

Down Hole Electromagnetic (EM) Surveys

Down hole electromagnetic (EM) surveys involve the arrangement of transmitting wires in various orientations or ‘loops’ on the ground. These are used to generate a primary magnetic field that induces an electrical current into conductive bodies. When the primary EM field is turned off, the induced field decays generating a secondary EM field and this data is captured via a receiver.

To acquire the data, several lengths of wire must be laid on the ground to provide the signal for measurement. A representative picture of the survey is shown in Figure 3. To assist with arranging the wires and collecting data, ATV vehicles are used to drive off-road (i.e. no direct clearing is required). Downhole EM survey involves a single loop of wire surrounding a drill hole. With this method a single LV or ATV is used to lay out and collect the loop. The downhole EM loops vary in size from 1000 m x 1000 m to 400 m x 400 m. A small temporary laydown will also be set-up in proximity to each survey area, no direct clearing will be undertaken to accommodate this, but rather a naturally open space will be utilised. Wherever possible, surveys are designed to use existing tracks and cleared areas so as to minimise impacts to the environment.

Figure 3.  Representative photos of how EM surveying is conducted.

Drilling

Drilling comprises RC, AC and DDH.  RC, AC and DDH may intersect groundwater, which is typically saline in the region and will require storage in temporary sumps. Where possible, drill sites will be located on existing grid lines created during previous drill programs, with additional clearing required for drill pads, sumps and, in some areas, widening of the grid line. Drill pads will be cleared of vegetation, which will be stockpiled at the end of the pad for use in rehabilitation. Topsoil will be stripped, where appropriate, to a nominal depth of 100 mm across the drill pad to reduce the risk of contamination and retain material for rehabilitation. The dimensions of the clearing and disturbance area for pads, sumps and tracks will be in accordance with the approved PoW.

A typical drill rig set up (DDH) is shown in Figure 4.

Figure 4.  A Diamond Drill Rig set up at an Exploration site.

Early stage Exploration Drilling

Early stage exploration drilling typically involves delineation of a target area on a broad drill-spacing, with drill traversed generally on lines greater than 100m apart. This density of drilling will be evaluated in the risk assessment section separately to the potential infill drilling stage of the exploration.

Infill Drilling

If positive results are encountered in the early stage exploration drilling, then infill drilling will be necessary to better define the resource. Closer drill pad spacing may be necessary in this stage of the exploration program. Given the increased cumulative impacts from the greater amount of drilling in this stage of exploration, it will be evaluated separately in the risk assessment section.  Further infill drilling to define a mineral resource may be required after this stage.

Clearing and disturbance

Where access for drilling is needed in a project area and if new access is required, the access track will be cleared by a rubber wheeled loader, using a raised-blade clearing method, along the pathway of minimum resistance, usually the most direct route, whilst utilizing any historical or rehabilitated track where available.  The clearing will avoid large or mature trees and dense stands of vegetation, as well as any threatened (and priority, where practicable) flora species, that will be identified in appropriate flora and vegetation surveys, completed before any disturbance is undertaken. The nominal width of exploration tracks is 3.5 m as shown in Figure 5.

To reduce the potential for impacts to project area from inadvertently introducing/spreading pathogens (e.g., Dieback) and weeds, strict hygiene protocols will be implemented for all vehicles and personnel entering the project area.  Specific hygiene management is discussed in Section 4.2.

Figure 5.  Access track clearance in Medallion Exploration

Camping

No camping other than temporary overnight fly camping will be undertaken on site. All other accommodation needs will utilise facilities at Forrestania. Any fly-camping will be on established tracks or areas of previous disturbance.

Hydrocarbons and other chemicals

A road licenced mobile diesel fuel tanker will be used to transport and store fuel on site and refuel vehicles and other equipment within the exploration area.  Small quantities of other hydrocarbons and chemicals will also be needed e.g. engine oil, and oils and greases for the drill rig.  Management of hydrocarbons and other chemicals is described in Section 4.4.

ENVIRONMENTAL RISK ASSESSMENT

A risk assessment has been completed to identify potential environmental hazard pathways and risk mitigation measures associated with the exploration program with reference to:

Types of exploration activities

Known environmental values of the area

Potential impacts from exploration activities

Risk level of exploration activities

The risk assessment uses a standardised framework to determine the inherent relative risk associated with each phase of exploration (Table 2). Management requirements are then determined based on the level of risk, as defined in Table 3. The results of the risk assessment, including the residual risk after implementation of management measures, are shown in Table 4.

Table 1.  Potential Hazard Pathway from exploration activities

| Potential Hazard Pathway | Detail |
| --- | --- |
| Loss of native vegetation | Disturbance and where necessary removal of native vegetation for access and work areas. |
| Loss of conservation significant flora | Disturbance and where necessary removal of native vegetation for access and work areas. |
| Loss of fauna habitat | Disturbance and where necessary removal of native vegetation for access and work areas. |
| Fauna death or injury | Vehicle impact and entrapment in drill holes or sumps. Improved access for feral animals |
| Soil disturbance, erosion, and compaction. | Vehicle movement and drilling |
| Disturbance to surface water drainage patterns | Creation of access routes, drill lines and drill sites along and across drainage lines. |
| Disturbance to surface water bodies. | Vehicle access to site. Exploration (geophysical or drilling) will not take place on surface water bodies |
| Introduction and spread of weeds. | Via movement of vehicles and other equipment. |
| Introduction and spread of dieback (and other potential pathogens). | Via movement of vehicles and other equipment. |
| Noise disturbance to people and fauna. | Vehicle movement and drilling |
| Dust disturbance to people and fauna. | Vehicle movement and drilling |
| Disturbance to culturally significant sites. | Vehicle movement and any ground disturbance |
| Increased risk of fire. | Introduction of potential sources of ignition (e.g. vehicles) into the project area during the drier months of the year. |
| Contamination of soil and water. | Spill or leak of hydrocarbons or other chemicals, saline or hyper-saline water during storage, handling, use or disposal. |
|  | Inappropriate disposal of waste, including food waste. |

Table 2.  Relative risk of exploration activities (DBCA 2011)

| Activity |  | Relative Risk |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 – Low | 2 – Low-Medium | 3 – Medium-High | 4 – High |
| Clearing | • • | No direct clearing (with graders or bulldozers). No indirect clearing (by off road driving) | - Off road light vehicle access with no blading or clearing - Minimal blading and no access construction | - Limited clearing of lowmedium value assets - Clearing on mainland outside southwest portion of Western Australia receiving rainfall <450 mm - Clearing vegetation with uniform weed distribution - Clearing on soils not highly susceptible to erosion | - Large scale clearing of low-medium value assets. - Any clearing of high – critical assets - Clearing on islands or in southwest portion of Western Australia with rainfall ≥450 mm - Clearing vegetation with patchy weed distribution - Clearing on soils highly susceptible to erosion |
| Drilling | • • • • | No drilling Soil sampling Hand augering Rock chip sampling | - Drilling does not encounter groundwater - Augering and / or vehicle mounted drilling. | - Drilling encounters groundwater - Small volumes of groundwater (e.g. 100s of L) - Air core, reverse circulation and diamond drilling, where clearance for pads and sumps is required in a wide spaced exploration program. | - Large volumes of groundwater - Saline groundwater - Cut and fill drilling pads • Air core, reverse circulation and diamond drilling, where clearance for pads and sumps is required in a close spaced exploration program. - Large clearing for deep drilling |
| Excavating |  |  |  |  | - Costeaning - Bulk sampling |
| Camping | • • | No Camping Fuel and chemicals only retained on vehicles | - Fly camp on mainland. - Fuel and chemical largely retained on vehicles | - Small permanent camp on mainland - Small fuel and chemical stores | - Large permanent camp on mainland and any camping on islands. - Medium to large fuel and chemical stores. |
| Vehicle Movement | • • | Vehicle access restricted to established roads and tracks (no offroad access) Helicopter based Survey | - Vehicle movements in dry soil conditions. • Off-road access: Only short-term evidence of tracks (1-2 growing seasons) as shrubs and trees can generally be avoided by vehicles (little pushing over of vegetation / sparse vegetation) - Vehicles operating in vegetation of uniform weed distribution. | Off road access: • Medium-term visibility of tracks due to unavoidable vehicle contact with woody plants (some pushing over of vegetation / medium density vegetation), or areas with high percentage cover of spinifex. - Vehicles operating in vegetation of varying weed distribution on mainland - Vehicle movements in wet soil conditions outside southwest portion of Western Australia receiving (rainfall <450 mm), or highly dispersive soil types. | Off-road access: • Long-term visibility of tracks due to susceptibility of soils to erosion or unavoidable vehicle contact with woody vegetation (some pushing over of vegetation / medium density vegetation), or areas with high percentage cover of spinifex. - Vehicles operating on any island or vehicles traversing pockets of serious environmental weed populations - Vehicle movements in wet soil conditions in southwest portion of Western Australia (rainfall ≥450 mm), or highly dispersive soil types. |

Table 3.  Management requirements as determined by risk ranking.

| Risk Rank | Risk Ranking Instruction |
| --- | --- |
| Low | Standard Exploration management procedures measures to be applied. |
| Low-Medium |  |
| Medium-High | Specific management procedures developed and implemented by senior management. Activities audited. |
| High | Specific mitigation measures and management procedures developed to address key risk items. All high-risk activities supervised by senior management and audited. Any deviation from the management procedures to result in immediate cessation of activity. |

Table 4.  Inherent and residual risk of proposed exploration phases.

| Exploration Program Stage | Inherent Risk Rating |  |  |  |  |  | Residual Risk Rating |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Clearing | Drilling | Excavating | Camping | Vehicle Movement | Clearing | Drilling | Excavating | Camping | Vehicle Movement |
| Stage 1 – Geophysical Surveys | 2 – Indirect clearing from 4WD vehicle movement along survey lines. | 1 – No drilling required | 1 – No excavating required | 1 – No camping required | 2 – off road access with short-term evidence of tracks and avoidance of shrubs and trees. | 1 – Indirect clearing from 4WD vehicle movement along survey lines. Vehicle movement minimised. | 1 – Vehicle mounted auguring. | 1 – No excavating required | 1 – No camping required | 2 – off road access with short-term evidence of tracks and avoidance of shrubs and trees. |
| Stage 2 – Early Stage Exploration Drilling | 3 - Limited clearing of medium value assets | 3 – Early Stage Exploration Drilling & RC drilling program, with pads and sumps required. Groundwater anticipated. | 1 – No excavating required | 1 – No camping required | 1 – Vehicle access restricted to established roads and tracks (no offroad access) | 2 - Limited clearing of medium value assets. Clearing kept to a minimum and management measures in place to minimise consequence. | 2 – Wide spaced DDH & RC drilling, groundwater encountered | 1 – No excavating required | 1 – No camping required | 1 – Vehicle access restricted to established roads and tracks (no offroad access) |
| Stage 3 – Infill Drilling | 3 - Limited clearing of medium value assets | 4 – Close spaced RC & DDH drilling program, with pads and sumps required. Groundwater anticipated. | 1 – No excavating required | 1 – No camping required | 1 – Vehicle access restricted to established roads and tracks (no offroad access) | 2 - Limited clearing of medium value assets. Clearing kept to a minimum and management measures in place to minimise consequence. | 3 – Close spaced drilling program, groundwater encountered, however, management measures in place to minimise impacts. | 1 – No excavating required | 1 – No excavating required | 1 – Vehicle access restricted to established roads and tracks (no offroad access) |

Revision 2 | Conservation Management Plan for | Date: 30/05/2025 	GENERAL 	Page 51

Custodian:  | UNLESS VIEWED ON THE DMS THIS DOCUMENT IS UNCONTROLLED

ENVIRONMENTAL RISK MANAGEMENT

Table 13 lists the environmental management aspects associated with proposed exploration program and the location of risk management measures.

Table 5.  Summary of risk management sections.

| Environmental Hazard Pathway management | Document section |
| --- | --- |
| Access and safety | Section 4.1 |
| Weed and Dieback management | Section 4.2 |
| Waste management | Section 4.3 |
| Hydrocarbon and other chemical management | Section 4.4 |
| Flora and Fauna management | Section 4.5 |
| Water management | Section 4.6 |
| Fire management | Section 4.7 |
| Dust, Light and noise management | Section 4.8 |
| Heritage and culture management | Section 4.9 |
| Rehabilitation | Section 0 |
| Induction and training | Section 4.10 |
| Incident reporting | Section 4.11 |

The following sections detail the environmental management measures that Medallion will implement and commitments that will be made to manage any potential environmental impacts in relation to exploration activities within the project area.

Access and safety

Suitable access from public roads and existing tracks has been identified within the project area. The following management actions will be undertaken to minimise impacts on the surrounding physical and biological environment:

Vehicle transit will only be via existing roads and tracks, including (but not limited to) paved public roads, unsealed (e.g. gravel roads), minor tracks, fence lines and fire breaks, provided they are in a suitable condition to be passable by an ordinary 4WD vehicle, drill rig and support vehicles.

Vehicles will be minimised from using unsealed roads during wet weather to ensure that the tracks are not damaged and minimize the risk of dieback spread, soil erosion and drainage siltation.

Vehicle speed will be regulated along the access tracks within the project areas to minimize risk of injury to animals and human and to reduce dust generation.

Signage and barricades if deemed necessary, will be posted at the access points and at the entrance to the work areas to prevent unauthorized entry.

Adequate PPE will be provided to all personnel for visibility and protection.

Weed and dieback management

Many weed species, as well as Phytophthora dieback, are present in the broader FGP project area. Three weed species with the potential to occur are listed as a Declared Pest in the Western Australian Organism List (WAOL) under the Biosecurity and Agriculture Management (BAM) Act 2007 and/or as a Weed of National Significance. The exploration activities have the potential to:

Introduce new weed species from outside the project area via vehicle or equipment tread, on site personal clothing and shoe treads.

Spread existing weeds from one section of the project area to another via vehicle or equipment tread, on site personal clothing and shoe treads.

Introduce Phytophthora dieback from outside the project area via vehicle or equipment tread, on site personal clothing and shoe treads.

Spread Phytophthora dieback from one section of the project area to another via vehicle or equipment tread, on site personal clothing and shoe treads.

The following measures will be implemented to minimise and mitigate the potential spread of and invasion by weeds and Dieback:

All vehicles will undergo a full weed and seed wash down at an appropriate facility prior to entry into the project area. A Weed and Seed and Vehicle Hygiene inspection form will be completed for each vehicle that will enter the project area and a new form completed as required on subsequent re-entries.

Vehicle transit will only be via existing roads and tracks, including (but not limited to) paved public roads, unsealed (e.g. gravel roads), minor tracks, fence lines and fire breaks.

Access will be minimized during periods of wet soil.

The number of entry points to the site will be minimised as part of exploration planning.

A weed species register will be maintained to inform the workers and to maintain records of weeds encountered. A guide detailing weed species that may occur will be made available to enable identification by field staff.

Weed hygiene will be according to the Medallion’s standard work procedure for weed and seed and vehicle hygiene.

Any work programs that are planned to occur in an area that has been identified as a dieback infected area (Figure 14) will be individually assessed to ensure that appropriate measures are put in place to minimise the potential spread of dieback within the FGP project area.  These measures will include:

Determining no go zones around areas of known infections and areas defined as uninfected,    thus eradicating the risk of spread.

Establishing Clean on Entry and/or Clean on Exit points if necessary.

Scheduling activities to minimise movement within and between areas, with areas of known infestation (i.e., Phytophthora Risk Area) drilled after areas of no known occurrence (i.e., Protectable Area).

General waste management

Waste expected to be generated would include domestic waste such as food waste, wrappers, plastic waste, soil/rock spoil generated from drilling activities, hydrocarbon contaminated items (e.g., oily rags) and general rubbish. The exploration activities have the potential to:

Incorrectly dispose of waste leading to soil, surface water or groundwater contamination.

May injure fauna via entrapment or ingesting waste.

Provide inadequate facility for waste disposal leading to littering into the local environment.

Attract feral animals.

The waste generated will be managed through proposed actions:

Food packaging and other general waste will be contained appropriately to avoid accidental littering and potential access by fauna.

Project team will be educated on appropriate disposal of cigarettes butts to avoid butt litter and fire risk.

Drill spoil will be backfilled into the drill sumps wherever possible.

All waste will be collected, segregated, and stored in properly constructed containers and removed to a licenced landfill facility or another disposal site in accordance with local council requirements and standard tenement condition.

Spill management

The use of fuels, oils, lubricants and other potentially contaminating chemicals during exploration activities has the potential to contaminate soil and water and to impact flora and fauna health. Medallion will implement the following management measures to reduce the potential impacts of hydrocarbons and other chemicals on the environment:

Minimal quantities of chemicals and hydrocarbons will be used during exploration activities and these will be stored within appropriate containers or within vehicle/s.

All the vehicles and equipment will be inspected daily for leaks of any type of fluid before entering the project area.

All refuelling or maintenance of vehicles must occur in an area cleared of native vegetation and stripped of topsoil.

All Exploration activities will have spill kits available and understand proper spill response.

Drill rigs will have a plastic liner underneath (rig nappy) to contain any minor spills.

Any spills will be managed as per the Exploration Hazardous Substance Clean Up and Disposal procedure reported to the Exploration Environment team and the necessary external regulators where required.

Flora and fauna management

- Flora and vegetation

Clearing of native vegetation during exploration activities can impact the environment by reduction of floral diversity and conservation significant species and impact the functioning of ecological communities through increased risk of soil erosion and loss of floral species. It is therefore important to reduce the impact of clearing as much as practical during exploration activities. Clearing may be required for construction of access tracks, drill pads and sumps. To manage impacts on soil and vegetation during exploration activities, Medallion will implement the following management measures:

Prior to any clearing of vegetation or ground disturbance, flora and vegetation surveys will be undertaken for the immediate area.  This will include a targeted flora and fauna survey as a minimum.

The Medallion Ground Disturbance Plan will be fully executed prior to any clearing taking place (as per the Exploration Management Plan).

The drill site set up will be detailed in the Medallion Ground Disturbance Plan.

Conservation significant flora that may occur in the area will be included in Medallion FGP Flora Identification Guide (Appendix 2) to aid identification by field staff.

Priority flora, identified from the field survey, will be taken into account during the earthworks planning stage and each individual plant will be avoided from disturbance where-ever possible.

Where Threatened flora (BC Act, EPBC Act) has been identified by flora and vegetation surveys, a 50 m exclusion zone will be established around each individual.

Where possible, clearing will be undertaken with a raised blade to minimise disturbance to roots and soil.

Where soil and overburden are being removed as part of major ground disturbing activities, the topsoil will be stripped and stockpiled separately, stored and returned to disturbed areas at the completion of exploration activities.

Cleared vegetation will be stockpiled for respreading at a later date, aiding in rehabilitation.

Existing tracks, fence lines and firebreaks will be used wherever possible to limit clearing.

Where practical, branches overhanging tracks which require removal will be pruned, rather than entirely removed, to minimise clearing.

Areas to be cleared will be clearly pegged or flagged and delineated in the field and marked on the site plan.  Persons undertaking clearing activities and the Medallion Exploration Representative will walk the area to be cleared prior to commencement of clearing, to confirm the area to be cleared.  All persons involved in clearing activities will be inducted on the importance of minimising vegetation clearing and disturbance.

Significant flora and habitat of significant fauna species will be avoided wherever possible; this will include retaining large trees and those with significant hollows.

Any suspected sightings of conservation significant species will be reported to the Medallion Environment team, GPS location and photographs will be recorded.

- Fauna

The exploration activities have the potential to pose the following risks on the fauna species likely to occur in the project area:

Mortality and morbidity due to collisions with vehicles.

Excessive noise resulting in altered behaviour.

Introduced fauna may compete with native species for resources.

Disturbance and destruction of fauna habitat from land clearing.

Fauna habitat fragmentation.

To reduce the risk of adverse outcome on native fauna species during the exploration activities, the following management measures will be implemented in addition to relevant measures discussed in the above sections:

All land clearing will be completed to the standard of Medallion Exploration Disturbance Management Plan.

Conservation significant fauna previously recorded, likely to occur or with the potential to occur in the management area will be included in the Medallion FGP Fauna Identification Guide, made available to staff to facilitate identification (Appendix 2).

All sightings of conservation significant species will be reported to the Exploration Environment team, GPS location will be recorded.

All drillholes will be temporarily capped upon the completion of drilling and rehabilitated as soon as practicable.

Any significant natural structure that are likely to provide habitat for the native fauna, such as hollow bearing trees, large trees and hollow logs will be avoided from disturbance wherever possible.

Exploration activities will be carried out during the daytime and avoided at night, where possible to reduce disturbance to fauna species.

Vehicles will travel at reduced speeds within the project area to minimise the likelihood of fauna mortality or morbidity associated with vehicle collisions. Any vehicle collisions involving conservation significant species will be immediately reported to the Exploration Manager and the Exploration Environment team.

Any significant fauna sighted or injured on collision will be notified to DBCA through Injured or Abandoned Fauna Notification Form pursuant to Biodiversity

Conservation Regulation 2018. All other injured fauna will be released back into wild or handed over to DBCA or suitable facility for rehabilitation.

Pets will not be allowed within the project area to prevent disturbances to conservation significant species and ensure that fauna are not subjected to predation or injury arising from interactions with domestic animals.

Staff members will not feed or interact with any native species unless intervention is strictly necessary to prevent injury to personnel or the animal in question.

All the domestic waste will be stored in container/bins with secure lid to deter fauna attraction.

Sumps are to be appropriately ramped to allow fauna egress. Sumps should be inspected regularly for fauna.

Carnaby’s Black Cockatoo impact mitigating measures:

All areas of proposed clearing will have targeted pre-clearance surveys to check for suitable black cockatoo habitat trees and evaluate them for suitable hollows.  For most species of tree used by Carnaby’s black cockatoos, this is a DBH of 300-500mm or larger.  These will be registered with a GPS location.

Clearing avoidance, wherever possible, of trees with a suitable diameter at breast height (DBH) to currently have hollows or likely to be able to form hollows used by Carnaby’s black cockatoo will be avoided

Clearing activities should, if practicable, be undertaken outside of the August to February breeding season to minimise risks to nesting birds.

If undertaking the proposed activities during this seasonal period, large trees with hollows (likely nesting trees) will be avoided from clearing with a 10m buffer.

Malleefowl impact mitigating measures:

All areas of proposed clearing will have targeted pre-clearance surveys to ensure that the any Malleefowl mounds within 200m of proposed clearing are identified.

All Malleefowl mounds will be avoided with a 50m buffer.

Prior to the Malleefowl breeding season (September to March), Malleefowl mounds in proximity to exploration activities will be evaluated for signs of activity and avoided with a minimum 50m exclusion zone, if signs of activity are detected.

Chuditch impact mitigating measures:

All areas of proposed clearing will have targeted pre-clearance to check for suitable chuditch habitat including hollow bearing trees and burrows.  These will be registered with a GPS location

Suitable Chuditch habitat including hollow bearing trees and burrows will be avoided from clearing or other disturbances

If suitable Chuditch habitat hollow bearing logs are identified within the proposed disturbance envelope, then they will be retained and moved into adjacent uncleared areas.

Water management

No significant water bodies or rivers coincide with areas of planned work, within the project area. The exploration work program is not anticipated to impact water quality by contamination from hydrocarbon, effluents directly draining onto the land and sediments from vehicle movement. The following management measures will however be implemented to reduce potential impacts on water environments:

Movement of vehicles on unsealed roads will not be conducted during and after significant rain events, to prevent erosion and silt generation in drainage. This may be informed by the local Shire opening nearby main roads.

Hydrocarbon management will be undertaken as discussed in Section 4.4 to prevent the risk of water contamination.

All drill water and cuttings will be contained in drill sump.

Polypipe will only be plumbed to the current drill pad/s to minimise the length of active polypipe.  Polypipe will be purged before any alterations are made to the line to ensure leakage is eliminated during relocation of the polypipe.

Water will be transported by a suitably licenced water-cart or polypipe to the drill site.

Fire management

Fire initiated as a result of exploration activities can result in destruction of flora, vegetation, fauna and fauna habitat. The following management measures will be implemented to reduce the risk of fire generation during exploration activities, as well as best manage any fire event not associated with the site activities:

All vehicles will be fitted with well-maintained exhaust systems to prevent the accumulation of combustible material against heat surfaces.

All vehicles will be fitted with an appropriate fire extinguisher.

Fire awareness and education on bush fires, including evacuation routes, in terms of emergency, will be imparted to the project team during induction.

All relevant fire control legislation, guidelines and instructions will be complied.

Any cigarette butts will be disposed of appropriately.

All the fire extinguishers and AFFF devices will be inspected periodically and serviced accordingly, if required.

Highly flammable substances will be appropriately stored during all exploration activities.

Total fire bans will be observed according to the advice from Department of Fire and Emergency Services (DFES) and in accordance with the Medallion Total Fire Ban Checklist.

No fires will be deliberately started. If a fire is noticed, DBCA and DFES will be contacted immediately.

Dust, light and noise management

Vehicles and drill rigs can generate dust, light and noise and therefore have the potential to negatively impact the environment. Dust can affect vegetation through significant accumulation on plants. Artificial light and noise can alter fauna foraging and reproductive behaviours, reduce reproductive success, and increase the risk of predation. The following management measures will be implemented to reduce potential impacts of dust and noise on the environment and people:

Vehicles will travel to signposted speeds on public roads.

Vehicles will travel at slow speeds on unsealed and minor roads to minimise both dust and noise.

Light pollution will be avoided wherever possible, any necessary lighting towers will be focused on the work area surface (drill pad) and thus no direct light will be dispersed into the surrounding bushland.

All drill rigs will use dust suppression systems where air drilling is used.

All vehicles will be regularly maintained to ensure they are operating optimally and not producing excessive noise.

Where necessary, shade cloth will be laid out at intersections and fixed to the ground using terra-pins to reduce degradation of the road surface and dusty conditions.

Heritage and culture management

Exploration activities shall comply with the Aboriginal Heritage Act 1972 at all times. Medallion will implement the Ground Disturbing Permit (GDP) process which considers aboriginal heritage management. Rehabilitation

Appropriate rehabilitation will ensure that the long-term impacts of exploration in the area will be minimal. DMPE rehabilitation requirements are that rehabilitated areas are made safe, stable, are non-polluting and have a demonstrated capacity to support a self-sustaining ecosystem similar to the surrounding natural environment. Medallion will comply with any tenement conditions set regarding rehabilitation and will follow the Medallion Exploration Disturbance Management Plan. Key rehabilitation measures to be implemented include the following:

Drill holes will be securely capped or plugged immediately at the completion of each drill hole.

Access routes, survey lines, drill pads, drill holes and sumps will be rehabilitated within twelve months of completion of each exploration program. However, if access routes are likely to be used in the near future to support further exploration, then Medallion will approach DMPE to discuss the need for retention and will apply to defer the requirements to rehabilitate these areas for a longer period (e.g. 6–12 months).

Plastic bags, grid pegs and other artificial debris and waste will be removed from site and disposed of appropriately.

Sump liner to be removed from drill sump.

Drill spoil will be emptied into sumps, or removed from site and disposed of at appropriately licensed facilities.

Drill holes will eventually be plugged below ground level (preferably with an appropriate conical plastic plug) and rehabilitated in accordance with DMPE guidelines.

Sumps will be backfilled and rehabilitated.

Compacted areas will be ripped along the original contour.

Stockpiled soil will be spread over disturbed area.

Stockpiled vegetation will be respread over finished surfaces.

Induction and training

The Exploration Manager will be responsible for ensuring that all personnel complete a site-specific exploration induction and are appropriately trained, and familiar with environmental procedures and responsibilities.

All site personnel and relevant office personnel will undergo a comprehensive environmental induction that will cover all the components of the onsite environmental management system with specific procedures to follow for matters such as clean-down points, exclusion zones, ground disturbance and vegetation clearing, waste storage and handling, hydrocarbon storage and handling, spill and other environmental hazard/incident response, reporting, and others.

Incident reporting

All exploration staff and contractors are responsible for ensuring that environmental and other incidents are reported immediately to the relevant Project Geologist. The Project Geologist will ensure that appropriate response actions are undertaken and will seek further environmental advice on the best course of action as needed. The Project Geologist is also responsible for ensuring that an Incident Report Form is completed and provided to the Exploration Manager. Environmental incidents will be investigated to determine their causes, and measures will be implemented to avoid, or at least minimise, recurrence.

ROLES AND RESPONSIBILITIES

| Roles |  | Responsibilities |
| --- | --- | --- |
| • • • Exploration Manager • |  | Ensure implementation of management measures outlined in the EEMP. Coordinate all exploration activities in accordance with applicable policies, procedures, legislative requirements, and management plans. Ensure adequate training and awareness are provided to the employees to fulfill their obligations outlined in this EEMP. Provide advice and assistance to exploration employees to ensure compliance with this EEMP. |
| • |  | Undertake incident investigations where required. |
| • |  | Manage the implementation of corrective and remedial actions arising from audits and incident investigations. |
| • • • Project Geologist |  | Ensure implementation of measures outlined in this EEMP. Ensure a copy of the EEMP available on site. Ensure vehicle and equipment hygiene is implemented. |
| • |  | Ensure dieback management plan is adhered to. |
| • |  | Report incidents and any audit outcomes to the Exploration Manager. |
| • |  | Conduct task specific inductions with relevant personnel. |
| Environmental • Team • |  | Provide suitable Environmental advice to project team. Conduct Environmental audits as necessary |
| • Senior Field • Assistant |  | Ensure all the activities comply this EEMP. Conduct task specific induction with relevant personnel. Ensure vehicle and equipment hygiene is implemented. |
| • |  | Ensure dieback management plan is adhered to. |
| • |  | Ensure incidents are reported. |
| • • All employees, • including • contractors • |  | Understand the employee responsibilities to conserving the environment as defined by this EEMP. Complete induction which includes information on dieback and its management. Dispose of all waste material appropriately as outlined in this EEMP. Keep to required existing tacks at all times. Provide assistance in implementing and maintaining impact minimisation programs when requested by the Senior Project Geologist. |
| • |  | Report incidents and non-compliance with this EEMP to the Project Geologist or the Exploration Manager. |

REPORTING AND COMMUNICATION

Internal

Internal reporting undertaken by Medallion includes:

- Notifications

- Regular Communication

- Reporting

- Incident Reporting

The Exploration Manager is responsible for external reporting.

External

External communication and reporting is required as per Table 6.

Table 6.  External reporting requirements.

| Agency | Contact Detail | Content | Timing |
| --- | --- | --- | --- |
| DMPE | Relevant officer at DMPE contacted directly | Rehabilitation reporting compiled by Exp Environmental team | At conclusion of rehabilitation activities |
| DMPE | Appropriate persons advised by Exp Environment team | PoW rehabilitation extension request compiled by Exp Environmental team | Annually during December |
| Emergency | 000 | Bushfire |  |
| DBCA | fauna@dbca.wa.gov.au | Injury/mortality of native fauna |  |

REFERENCES

Beard, J.S.,.1990. Plant Life of Western Australia, Kangaroo Press Pty Ltd, NSW.

Beecham, B., & Danks, A. 2001. A Biodiversity Audit of Western Australia’s 53 Biogeographical Region in 2001; Mallee 2 (MAL2 –Western Mallee subregion) pp 435-465, Department of Conservation and Land Management, November 2001

Botanica Consulting. 2024. South Iron Cap Project Flora and Fauna Assessment. Prepared for Medallion Limited February 2024.

Botanica Consulting. 2025.  Forrestania Project.  Reconnaisance Flora and Basic Funa Survey.

Prepared for Medallion Limited, March 2025.

Bureau of Meteorology [BoM] (2025). Climate Data. Bureau of Meteorology. Available: http://www.bom.gov.au/climate. Accessed 18th February 2025.

DBCA. 2019. 2018 Statewide Vegetation Statistics (formerly the CAR Reserve Analysis). Department of Biodiversity, Conservation and Attractions.

Department of Conservation and Land Management (CALM). Unknown. The Chuditch. Pamphlet distributed by the current equivalent agency, the Department of Biodiversity, Conservation and Attractions.

DBCA (2025a). Priority/ Threatened Flora Database Search [Ref: 26-0325FL]. Obtained from Department of Biodiversity, Conservation and Attractions. March 2025.

DBCA (2025b). NatureMap Database search. Obtained from Department of Biodiversity, Conservation and Attractions, March 2025.

DBCA (2025c). Priority/ Threatened Fauna Database Search [Ref: 14-0325FA]. Obtained from Department of Biodiversity, Conservation and Attractions. March 2025.

DBCA (2025d). Priority/ Threatened Ecological Communities Database Search [Ref: 08-0325EC].

Obtained from Department of Biodiversity, Conservation and Attractions. March 2025.

DPIRD. 2019. Pre-European Vegetation (DPIRD_006) Department of Primary Industries and Regional Development, Western Australia, 24 July 2019

EPA. 2000. Position Statement No. 2 Environmental Protection of Native Vegetation in Western Australia, Environmental Protection Authority

Government of Western Australia. 2022. Dandjoo Biodiversity Data Platform. Accessed 28/09/2023 from: Dandjoo biodiversity data platform | Biodiversity Information Office

Groom, 2010. Artificial hollows for Carnaby’s black cockatoo.  Project report for the Department of Environment and Conservation, August 2010.

Mattiske, EM and Havel JJ. 1998. Vegetation Mapping in the South West of Western Australia and Regional Forest Agreement vegetation complexes. Map sheets for Pemberton, Collie, Pinjarra, Busselton- Margaret River, Mt Barker, and Perth, Western Australia. Scale 1:250,000. Department of

Conservation 	and 	Land 	Management, 	Perth 	Accessed 	22/09/2023 	from: https://catalogue.data.wa.gov.au/dataset/vegetation-complexes-swf-50k

Soderquist, T. R. 1988.  The Ecology of Chuditch (Dasyurus geoffroii) in the Jarrah Forest: A Summary of Facts Relevant to Management.  Department of Conservation and Land Management October 1988.

Tille, P. 2006. Soil Landscapes of Western Australia’s Rangelands and Arid Interior, Department of

Agriculture and Food Western Australia

Western Australian Herbarium.1998–. Florabase—the Western Australian Flora. Department of Biodiversity, Conservation and Attractions. https://florabase.dbca.wa.gov.au/ (Accessed 27 September 2023).

York, M. 2005.  A Study of the Effects of Probait on Chuditch, Dasyurus geoffroii, within the Julimar Conservation Park with notes on the general ecology. Thesis Submitted in Partial Fulfillment of the Requirements for the Award of Bachelor of Science (Biological Science) with Honours. Edith Cowan University.

Revision Tracking

| Revision | Author | Section | Description of Change |
| --- | --- | --- | --- |
| 0 | Jenny Safstrom |  | Document Issued for First Use |