Clearing Permit Decision Report

# Application details

## Permit application details

| Permit application No.: | 74/1 |
| --- | --- |
| Permit type: | Area Permit |

## Proponent details

| Proponent’s name: | MR Luigi & Albina Cordisco |
| --- | --- |
| Postal address: | 93 Brisbane St Perth WA 6000 |
| Contacts: | Phone: 92278077 |
|  | Fax: |
|  | E-mail: |

## Property details

| Property: | Lot 17 on Plan 8480 (House No. 41 Knight Gnangara 6065) |
| --- | --- |
| Local Government Area: | City of Wanneroo |

## Application

| Clearing Area (ha) | No. Trees | Method of Clearing | For the purpose of: |
| --- | --- | --- | --- |
| 4 |  | Mechanical Removal | Horticulture |

# Site Information

## Existing environment and information

### Description of the native vegetation under application

| Vegetation Description | Clearing Description | Vegetation Condition | Comment |
| --- | --- | --- | --- |
| Beard veg type - 1949 low woodland; banksia on low sandhills, swamps and swales with teetree and paperbark. Heddle veg type - Bassendean Complex North/Transition - low open forest and low woodland of Banksia species, E. todtiana on a series of sand dunes. | Vegetation to be cleared is Banksia low open woodland with a dense shrub layer. The dominant species is Banksia with patches of E. todtiana. | Very Good: Vegetation structure altered; obvious signs of disturbance (Keighery 1994) | Banksia low open woodland with a dense shrub layer. The dominant species is Banksia with patches of E. todtiana. The vegetation is in very good condition with good numbers of understorey species and minimal weed numbers (site visit report). |

# Assessment of application against clearing principles

#### (a)	Native vegetation should not be cleared if it comprises a high level of biological diversity.

| Comments | Proposal may be at variance to this Principle |
| --- | --- |
|  | No information was provided or available to enable an assessment against this Principle (CALM 2004). |
| Methodology |  |

#### Native vegetation should not be cleared if it comprises the whole or a part of, or is necessary for the maintenance of, a significant habitat for fauna indigenous to Western Australia.

| Comments | Proposal may be at variance to this Principle |
| --- | --- |
|  | CALM identify a medium probability of the proposed clearing to be at variance with this Principle. There are a high number of specially protected and priority fauna found in the area, strongly indicating a need for a fauna survey. Additionally, clearing this area will significantly diminish the connection between the remnants to the north and south (CALM 2004). |
| Methodology | CALM 2004; CALM Threatened and Priority Fauna Database. |

#### (c)	Native vegetation should not be cleared if it includes, or is necessary for the continued existence of, significant flora.

| Comments | Proposal may be at variance to this Principle |
| --- | --- |
|  | CALM identifies that there is a medium probability of the proposed clearing to be at variance with Principle (c). Two populations of Declared Rare Flora are known from the local area (10km radius) in addition to records of one Priority 1, four Priority 2, nine Priority 3 and five Priority 4 species. CALM advises that a flora survey, undertaken at the appropriate time of the year, is required to determine if Caladenia huegelii is present on the property and, if so, the possible impact the clearing will have on this species. |
| Methodology | CALM 2004; CALM Threatened Flora Data Management System; CALM Herbarium Specimen Collection Database. |

#### (d)	Native vegetation should not be cleared if it comprises the whole or a part of, or is necessary for the maintenance of a significant ecological community.

| Comments | Proposal is at variance to this Principle |
| --- | --- |
|  | There is a high probability of the proposed clearing to be at variance with Principle (d) (CALM 2004). There is a high likelihood of community type 20a occurring at the site given the proximity of 12 other occurrences nearby. CALM advises that a survey be undertaken at the appropriate time of year to search for community type 20a. |
| Methodology | CALM Threatened Ecological Community Database. |

#### (e)	Native vegetation should not be cleared if it is significant as a remnant of native vegetation in an area that has been extensively cleared.

| Comments | Proposal is not at variance to this Principle |  |
| --- | --- | --- |
|  | The Bioregion and Shire have been cleared fairly extensively, with 41.8% and 57.6% of the pre-European extent of native vegetation remaining respectively (Shepherd et al. 2001). These values are above the 30% benchmark target (Department of Natural Resources and Environment, 2002; EPA, 2000). The Beard vegetation type (1949) has been extensively cleared with only 25.6% of the pre-European extent of native vegetation remaining. However, the more fine-scale Heddle mapping indicates that the vegetation type for the area under application (Bassendean Complex North/Transition) is still well-represented with 92.3% of the pre-European extent remaining. Pre-European Current Remaining Conservation Reserves/CALM- area (ha) extent (ha) %* status** managed land, % veg IBRA Bioregion - Swan Coastal Plain 1,498,297*** 626,512 41.8 Depleted Shire - City of Waneroo 78,809 45,361 57.6 Least concern Beard veg type - 1949 132,958 34,012 25.6 Vulnerable 24.4 Heddle veg type - Bassendean Complex North/Transition 17,675 16,308 92.3 Least concern 62.7 * (Shepherd et al. 2001) ** (Department of Natural Resources and Environment 2002) *** Area within the Intensive Landuse Zone |  |
| Methodology | Shepherd et al. 2001; Heddle et al. 1980 |  |

#### (f)	Native vegetation should not be cleared if it is growing in, or in association with, an environment associated with a watercourse or wetland.

| Comments | Proposal is not at variance to this Principle |
| --- | --- |
|  | The property is not located on a watercourse or wetlands. East of the property: The property is located between two Environmental Protection (Gnangara Mound Crown Land) Policy (EPP) Areas (360m and 600m). The southern EPP Area (360m away) is also a Bush Forever site (193 - Gnangara Lake and adjacent bushland). West of the property: The area under application is 60m from an area of mapped Multiple Use Dampland (which has been cleared) and 300m from a Resource Enhancement Dampland. |
| Methodology | GIS databases: EPP Lakes, Wetlands and Areas, Hydrography and rivers databases. |

#### (g)	Native vegetation should not be cleared if the clearing of the vegetation is likely to cause appreciable land degradation.

| Comments | Proposal may be at variance to this Principle |
| --- | --- |
|  | The site has a moderate to low risk of acid sulphate soils (ASS) occurring at a depth of >3m; no risk of occurrence <3m from soil surface. However the area is somewhat marginal and a 1m change in the watertable shifts the area from moderate to high. If the proponent intends to carry out any excavations associated with the proposed land use, this issue could become important. Additionally, horticulture will leave the soil exposed between crops, which will increase the risk of ASS. Department of Agriculture (DAWA) report indicates potential for increased waterlogging down slope of the site, eutrophication, and wind erosion to occur following clearing. However they note that there are numerous market gardens in the area and that these issues could be managed to minimise the problems. |
| Methodology | Acid Sulphate Soil risk map (GIS) - Swan Coastal Plain; DAWA advice. |

#### (h)	Native vegetation should not be cleared if the clearing of the vegetation is likely to have an impact on the environmental values of any adjacent or nearby conservation area.

| Comments | Proposal is not likely to be at variance to this Principle |
| --- | --- |
|  | The proposed clearing of 4 ha of native vegetation will not significantly affect the surrounding CALM managed lands (CALM 2004). |
| Methodology | CALM 2004 |

#### (i)	Native vegetation should not be cleared if the clearing of the vegetation is likely to cause deterioration in the quality of surface or underground water.

| Comments | Proposal is at variance to this Principle |
| --- | --- |
|  | The property is located in a Priority 2 Water Protection Area (Mirrabooka Groundwater Area) and horticulture is not compatible with this zoning. The previous owners were refused a water licence and it is not likely that the new owners will be granted a licence. |
| Methodology | Water Licencing, Department of Environment. |

#### (j)	Native vegetation should not be cleared if clearing the vegetation is likely to cause, or exacerbate, the incidence of flooding.

| Comments | Proposal is not at variance to this Principle |
| --- | --- |
|  | Scale of clearing is unlikely to have a noticeable effect on flood peaks and/or duration. |
| Methodology |  |

#### (k)	Planning instrument or other matter.

| Comments | No comment made |
| --- | --- |
| Methodology |  |

# Assessor’s recommendations

The recommendations of the Department of Environment to the CEO of the Department should be made consistent with the outcomes of the assessment by each of the agencies.  Any conditions on the approval should also be outlined.  These may be developed in consultation with such other agencies as required.

| Purpose | Method | Applied area (ha)/ trees |  | Decision |  | Comment / recommendation |
| --- | --- | --- | --- | --- | --- | --- |
| Horticulture | Mechanical Removal | 4 |  |  | Refuse | The proposal is at variance with Principle (d). The proposal may also be at variance with Principles (b) and (c); a flora survey and fauna habitat site analysis would be required. There is also a reasonable risk that the proposal is at variance with Principle (g) due to the risk of Acid Sulphate Soils, which will be exacerbated by horticultural production. Finally, the proposal is at variance with Principle (i) and application for a ground-water license is unlikely to be successful. |

# References

Department of Natural Resources and Environment (2002) Biodiversity Action Planning. Action planning for native biodiversity at multiple scales ; catchment bioregional, landscape, local. Department of Natural Resources and Environment, Victoria. Keighery, BJ (1994) Bushland Plant Survey: A Guide to Plant Community Survey for the Community. Wildflower Society of WA (Inc). Nedlands, Western Australia. Shepherd, D.P., Beeston, G.R. and Hopkins, A.J.M. (2001) Native Vegetation in Western Australia, Extent, Type and Status. Resource Management Technical Report 249. Department of Agriculture, Western Australia.